# CSI Monitor User Guide with Automation (AP-Client & AP-AP)

# Introduction to CSIMON

The CSIMON (Channel State Information Monitor) command is used to enable, configure, and control CSI data capture in Broadcom-based Wi-Fi devices. It allows users to collect channel characteristics by triggering specific frame exchanges and extracting CSI from received ACK frames.

Key CSIMON commands and arguments:
- csimon enable / disable: Starts or stops CSI monitoring
- csimon add <client_mac> <interval_ms>: Adds a client for CSI capture
- csimon del <client_mac>: Removes a client from monitoring
- csimon addap <ap_mac> <SSID> <interval_ms>: Configures AP-to-AP CSI
- csimon state: Displays CSI monitor status and counters
- dump csimon: Shows detailed statistics per client/AP
- csi_deci <factor>: Sets decimation (tone sampling rate)

The CSI records are delivered to user space through a netlink socket and can be consumed using applications such as csimond.

# Example Interface Mapping

5GHz Radio Interface: aruba000

6GHz Radio Interface: aruba200

# Part 1: AP to Client CSI Measurement

Example for 5GHz (aruba000):

wl -i aruba000 csimon enable

wl -i aruba000 assoclist

wl -i aruba000 csimon add <client_mac> 1000

csimond 22 0 1000 &

wl -i aruba000 csimon state

# Part 2: AP to AP CSI Measurement

Example for 6GHz (aruba200):

wl -i aruba200 csimon enable

wl -i aruba200 csimon addap <ap_mac> <SSID> 1000

csimond 24 0 1000 &

wl -i aruba200 csimon state

# Automation Scripts for CSI Experiments

Experiment configuration file (experiment_config.env):

IFACE=aruba000
NETLINK_ID=22
MODE=CLIENT
CLIENT_MAC=64:BC:0C:66:CF:C6
AP_MAC=AA:BB:CC:DD:EE:FF
AP_SSID=MyAP
INTERVAL_MS=1000
DECIMATION=0
CSIMOND_FREQ=1000
CSIMOND_RECORD_SIZE=0
LOG_DIR=/tmp/csi_logs
TAG=exp1

Main automation script (run_csi_experiment.sh):

#!/bin/bash
source ./experiment_config.env
mkdir -p ${LOG_DIR}
wl -i ${IFACE} csimon enable
wl -i ${IFACE} csi_deci ${DECIMATION}

if [ "${MODE}" == "CLIENT" ]; then
  wl -i ${IFACE} csimon add ${CLIENT_MAC} ${INTERVAL_MS}
else
  wl -i ${IFACE} csimon addap ${AP_MAC} ${AP_SSID} ${INTERVAL_MS}
fi

csimond ${NETLINK_ID} ${CSIMOND_RECORD_SIZE} ${CSIMOND_FREQ} &

Monitoring script (monitor_stats.sh):

#!/bin/bash
while true; do
  wl -i aruba000 csimon state
  wl -i aruba000 dump csimon
  sleep 2
done

# Important Notes

Clients must be associated for CSI capture.

AP-AP mode supports one peer at a time.

Settings reset after reboot.

Always run csimond to avoid data drops.